| Microcontroller- ESP32(WROOM-32) | SBC-Raspberry Pi 4 Model B |
| --- | --- |
| Processor- 32bit dual core Xtensa LX6 processor, 240MHz clock speed | BCM2711, Quad-core Cortex-A72 (ARM v8) 64-bit SoC @ 1.5GHz or 1.8GHz |
| SRAM-520KB | SRAM- LPDDR4 1GB,2GB,4GB or 8GB variants |
| Power source; 3.3V | Power source: 5V |
| General application: smart home devices, wearables, weather sensors etc | General applications: Low cost desktop computing, NAS(Network Attached Storage) hosts, web servers etc. |
| OS: Real Time Operating System(Bare metal firmware) | OS: General Purpose Operating System(Linux based Raspberry Pi OS) |
| 32 bit dual core architecture with speed in MHz | 64 bit quad core architecture with speed in GHz |
| Boot up time Instantaneous | Boot up time:15-40 seconds to load OS files |
| Exremely low power consumption | High power consumption |
| No native video/HDMI ports, interacts via pins | Dual 4K HDMI, USB 3.0, Gigabit Ethernet |

A microcontroller is designed to control a single physical machine or product with absolute timing precision.

A single-board computer is designed to run multiple user applications like a standard desktop computer.